Exercise 1- Differentiator and Integrator

Tasks:

1.) Measure the amplitude- frequency characteristics of differentiator and integrator

2.) Measure the response of differentiator and integrator when the applied signal is a periodic square wave for different parameters of input periodic signal (amplitude, DC, frequency- τ < T, τ ≈ T, τ > T)

Theory:

http://www.physclips.unsw.edu.au/jw/RCfilters.html

http://www.st-andrews.ac.uk/~www_pa/Scots_Guide/experiment/

Exercise 2- V-A characteristics of diode

D- general, LED, Zener diode (ZD)

Tasks:

1.) Measure the V-A characteristics of diode (for input voltage u1)

2.) Find the quiescent operating point (Q- point)

Theory:

- Lectures

Exercise. 3- Half- wave rectifier

Tasks:

1.) Measure the static transfer characteristic of half- wave rectifier

2.) Measure the output waveform of half- wave rectifier in time domain with (and without) capacitor C, for different parameters of input periodic signal (frequency)

Theory:

- Lectures

Exercise 4- Transistor Characteristics curves

Tasks:

1.) Measure the input characteristic of transistor (Emitter- base)

2.) Measure the output characteristic of transistor (Collector- emitter), where base current is a parameter

3.) Find the quiescent operating point (Q- point)

Theory:

- Lectures

Exercise 5- Differential Amplifiers

Tasks:

1.) Measure the differential- mode gain AD on the output of differential amplifiers in time domain, for different parameters of input periodic signal (frequency)

2.) Measure the task 1 for different values of current I0

3.) Measure the DC transfer characteristic U3= f(U12)

Theory:

- Lectures

Exercise 6- Operational Amplifiers circuits

Tasks:

1.) Measure the output waveform of inverting amplifier in time domain, for different parameters of input periodic signal (frequency) and different values of R2. Find the gain of the circuit for particular values of R2.

2.) Measure the output waveform of noninverting amplifier in time domain, for different parameters of input periodic signal (frequency) and different values of R2. Find the gain of the circuit for particular values of R2.

3.) Measure the DC transfer characteristic of inverting and noninverting amplifier for particular values of R2.

Theory:

- Lectures

Pokyny pre vypracovanie referátov. Každé meranie musí obsahovať:

A) Prednú stranu s:

názvom školy (TUKE),

názvom katedry (KEMT),

názvom merania (napr. Zapojenia s OZ),

menom kto meranie vypracoval a menami ktorí meranie uskutočnili,

šk. r. (2008/2009),

skupinou (napr. 1),

št. odborom (napr. VTI)

B) Ďalšie strany referátu musia obsahovať:

- Teoreticky rozbor merania/jednotlivých úloh (na základe teórie z prednášok, spoločných cvičení alebo inej dostupnej odbornej literatúry).

- Úlohy (všetky, ktoré sú uvedené v zadaní).

- Schémy zapojení jednotlivých úloh (všetkých, ktoré sú uvedené v zadaní LEN v OrCADe).

- Postup jednotlivých meraní (všetkých, ktoré sú uvedené v zadaní- tu uviesť, ktoré úlohy neboli zrealizovane a z akého dôvodu).

- Tabuľky a grafy (v EXCELy) nameraných hodnôt (len zrealizovaných úloh).

- Simulácia (v SPICE) všetkých úloh, ktoré sú uvedené v zadaní.

- Zhodnotenie jednotlivých meraní (u zrealizovaných meraní porovnať namerané výsledky s teoretickými a odsimulovanými, u nezrealizovaných meraní porovnať odsimulované výsledky s teoretickými). Tu uviesť:

  - chyby (napr. -relatívne, absolútne., štatistické, ...),

  - odchýlky od ideálnych- teoretických výsledkov,

  - nelinearity, ....

- Súpis použitých prístrojov.

- Použité elektronické prostriedky (napr. fotky uskutočnené mobilným telefónom Nokia 6300).

- Použitá literatúra.

C) Hodnotenie referátov:

  - Každý referát je hodnotený 4B. Celkovo je možné za referáty získať 16B.

  - Vypracovaný referát odovzdáva každý študent samostatne za svoju osobu.

  - Merania sa môžu odovzdávať priebežne, najneskôr však v 11. týždni. Po tomto termíne bude za každý odovzdaný referát stiahnutých 50B. Ďalšie podmienky u cvičiacich.

  - Podmienkou k udeleniu zápočtu je aby každý študent mal odovzdané minimálne 2 referáty. V prípade nesplnenia tejto podmienky bude študent riešený individuálne.

Ďalšie pokyny cvičiaci: Ing. Bugár, Ing. Lojka, Ing. Papco

Pre ak. rok 2008/2009:

GALAJDA